# Home Improvement Application Packet

The Ranch Community Association — Architectural Review

Property: 12344 Passerine Court, Rancho Cordova, CA — Lot 46, The Ranch Village 2

Project: Sierra Fiberglass “Bermuda” swimming pool (14' × 30'), concrete pool deck, and backyard landscaping

Prepared: July 2026    Owner: ______________________

## Packet Contents

1. Submission Checklist

2. Application Form Draft (copy onto the official HOA form)

3. Site Plan (scale drawing)

4. Landscape Plan — plant locations & species

5. Concrete Specifications, Setbacks & Drainage Statement

6. Pervious Surface Worksheet

7. Photographs of Existing Conditions

Attachments (separate files): product brochure PDF, approved plot plan PDF

Submit to: Landmark Limited, Attn: Home Improvement Dept., 1731 E. Roseville Pkwy., Suite 100, Roseville, CA 95661  ·  homeimprovements@landmarklimited.net  ·  (916) 746-0011

# 1. Submission Checklist

Checked items are included in this packet. Unchecked items must be completed by the homeowner before submitting.

## Included in this packet

☑  Landscape plan — location & species of all plantings and materials

☑  Site plan / detailed drawing — existing structure, fence lines, property lines, improvements

☑  Color, finish and dimensions of proposed concrete

☑  Concrete setbacks from property lines + drainage statement

☑  Pervious-surface worksheet (lot math prepared; needs your flatwork measurements)

☑  Color photographs of the area from several angles

☑  Product brochure (Sierra Fiberglass Bermuda PDF)

☑  Application form draft answers

## Homeowner to complete

☐  Print and sign the official application form (name, phone, dates, owner signature)

☐  Neighbor acknowledgment signatures (Lots 45 & 47 and rear neighbors)

☐  Measure existing flatwork (driveway, front walk, rear walkway) and complete the worksheet blanks

☐  Concrete color/finish sample chip from contractor

☐  City of Rancho Cordova building permit (pools require one)

☐  Photos of any work already completed (N/A if none started)

☐  Mail or email the complete package to Landmark Limited; allow up to 45 days

Reminders: work may NOT begin before written approval. Incomplete applications are returned. Construction hours after approval: 8:00 a.m.–6:00 p.m.

# 2. Application Form Draft

Copy these answers onto the official Home Improvement Application form.

| Field | Enter |
| --- | --- |
| NAME | (your name) |
| DATE | (submission date) |
| ADDRESS | 12344 Passerine Court, Rancho Cordova, CA |
| LOT NO | 46 |
| PHONE (H)/(W) | (your numbers) |
| E-MAIL | jay@quampy.com |
| PROPOSED COMPLETION DATE | (typ. 8–12 weeks from permit) |

## Boxes to check

Swimming Pool/Spa/Solar Panels

Landscaping backyard (plant location & species attached)

Driveway/Walkways (covers new concrete deck flatwork)

Materials: Concrete — write in “Fiberglass pool shell (Sierra Fiberglass — Bermuda)”

Same color/type as house: No — “Concrete matches existing builder walkway (natural gray, broom finish)”

## Additional Comments (suggested text)

Install a Sierra Fiberglass “Bermuda” swimming pool (14' wide × 30' long × 3'6"–6' deep, freeform) in the rear yard, centered between side fences. Pool is surrounded by a new 3'-wide broom-finish concrete deck (36' × 20' gross footprint) that ties into the existing rear walkway. Concrete is set back 3' from the rear property line per HOA requirement; the setback strip is planted as a privacy screen (Podocarpus gracilior at 4' o.c.) with accent plantings (kangaroo paw, salvia, lantana, lomandra) flanking the deck. Existing drainage pattern is unchanged; deck slopes away from pool and house to pervious landscape. See attached site plan, landscape plan, concrete specifications, pervious-surface worksheet, photographs, product brochure, and builder plot plan. A City of Rancho Cordova building permit will be obtained prior to construction; a copy can be provided upon issuance.

# 3. Site Plan

Scale drawing — pool placement, setbacks, fence lines, and planting areas. Property lines and recorded setbacks per the attached approved plot plan.

# 4. Landscape Plan — Plant Locations & Species

Backyard only (53' × 25'). Locations shown on the Site Plan. All selections are USDA Zone 9b / Sunset 14 appropriate (Sacramento area).

## Area A — Privacy screen (3' setback strip, rear fence)

| Qty | Plant | Botanical name | Install size | Spacing | Maintained size |
| --- | --- | --- | --- | --- | --- |
| 12 | Fern Pine (columnar screen) | Podocarpus gracilior | 15-gal | 4' o.c. | 8–10' H × 3–4' W |

Alternates at same spacing: Photinia × fraseri, or Green Giant Arborvitae. Podocarpus recommended near pools (evergreen, low litter).

## Area B — Accent color pockets flanking the deck

| Qty | Plant | Botanical name | Install size | Spacing | Bloom |
| --- | --- | --- | --- | --- | --- |
| 8 | Kangaroo Paw | Anigozanthos 'Bush Ranger' / flavidus hybrids | 1-gal | 30" o.c. | Red/orange, spring–fall |
| 6 | Autumn Sage | Salvia greggii (red & pink) | 1-gal | 30" o.c. | Spring–fall |
| 6 | Lantana 'New Gold' | Lantana × hybrida | 1-gal | 3' o.c. | Gold, spring–frost |
| 6 | Dwarf Mat Rush | Lomandra longifolia 'Breeze' | 1-gal | 30" o.c. | Evergreen grass |

## Area C — Remaining open yard

Decomposed granite or river rock over landscape fabric (pervious), or drought-tolerant no-mow fescue.

## Materials & Irrigation

Bed mulch: 3" shredded bark or ¾" crushed rock (pervious)

Bed edging: steel or concrete mow strip

Irrigation: automatic drip to all plantings, per Rancho Cordova Municipal Code 23.716.060

No front-yard changes; no plantings within the recorded P.U.E.

# 5. Concrete Specifications, Setbacks & Drainage

## Specifications

| Item | Specification |
| --- | --- |
| Overall deck footprint | 36' × 20' (720 sf gross) |
| Pool shell cut-out | approx. 380 sf (Bermuda 14' × 30' freeform) |
| Net new concrete flatwork | approx. 340 sf |
| Thickness | 4" over compacted aggregate base (per pool contractor) |
| Color | Natural gray — matches existing builder walkway |
| Finish | Medium broom (slip-resistant), matches existing flatwork |
| Coping | Pre-cast concrete, buff/cream tone |
| Joints | Tooled control joints, nominal 9' grid |

A physical color/finish sample chip from the concrete contractor accompanies this application.

## Setbacks from property lines

| Direction | Distance to concrete edge |
| --- | --- |
| Rear (north fence) | 3' 0" — HOA setback, kept as planted strip |
| Side (east) | 8' 6" |
| Side (west) | 8' 6" |
| House side | Ties into existing walkway; 2' clear at house wall |

Pool water edge is a minimum of 6' 0" from the rear property line (3' setback + 3' deck).

## Drainage statement

Existing drainage pattern is NOT altered; downspouts remain on builder underground drains

Deck finished at grade, sloped min. 2% away from pool and house onto pervious landscape

3' rear setback strip remains a pervious planted bed; no runoff directed at neighboring lots

Any deck drains discharge to the existing yard drainage system

To be verified against the pool contractor's final engineering/grading plan

# 6. Pervious Surface Worksheet

CC&R rules: max 75% of total lot non-pervious; max 40% of front yard non-pervious.

## Known values (from approved plot plan)

| Item | Value |
| --- | --- |
| Lot area | 5,565 sf |
| House footprint | 2,301 sf |
| Builder-stated lot coverage | 41.3% |
| New pool + deck (this application, counted fully non-pervious) | 720 sf |

## Measure and fill in

| Item | Measured sf |
| --- | --- |
| Driveway |  |
| Front walk / porch flatwork not in house footprint |  |
| Existing rear walkway / patio |  |
| Other existing flatwork (side pads, trash pad) |  |
| TOTAL existing flatwork (D) |  |

## Calculation

Total non-pervious = 2,301 + D + 720 = 3,021 + D

Percentage = (3,021 + D) ÷ 5,565     75% limit = 4,174 sf  →  D must be ≤ 1,153 sf

Heads-up: a 3-car driveway (~550–650 sf) plus existing walkways can approach this limit. Measure before submitting. If over 75%: reduce deck aprons, use permeable pavers for part of the deck, or ask Landmark whether pool water surface may be excluded.

## Front yard

No front-yard changes are proposed — the front-yard non-pervious ratio is unchanged by this application.

# 7. Photographs of Existing Conditions

Rear yard as it exists today (bare dirt, 6' wood fence, builder walkway), three angles.

View 1 — from side yard looking across the rear fence line

View 2 — from the patio looking north across the yard

View 3 — along the yard from the east end (walkway and A/C at right)